# 🖥️ Darby Linux — PowerPC Mac Hardware Compatibility List

Complete list of supported PowerPC Macs (1999–2006) and their Linux hardware compatibility status.

All models listed are New World ROM Macs that can boot Linux via Yaboot/Open Firmware. Older Old World ROM Macs (pre-1999) are not supported by this project.

## 📋 Table of Contents

- Compatibility Rating Legend

- iBook Series

- PowerBook Series

- iMac Series

- Power Mac Series

- Mac mini

- eMac

- Xserve

- Driver Availability Matrix

- Known Issues by Model

- Recommended Configurations

## Compatibility Rating Legend

| Rating | Meaning |
| --- | --- |
| 🟢 Excellent | All hardware works out of the box or with minimal configuration |
| 🟡 Good | Most hardware works; minor issues with 1–2 components |
| 🟠 Fair | Core functionality works but some hardware requires workarounds |
| 🔴 Limited | Significant hardware issues; experienced users only |
| ⚫ Unsupported | Known to not work or not tested |

## 💻 iBook Series

### iBook G3 (Clamshell) — 1999–2001

| Model | CPU | RAM (Max) | GPU | WiFi | Rating |
| --- | --- | --- | --- | --- | --- |
| iBook G3/300 (Tangerine) | G3 300 MHz | 576 MB | ATI Rage Mobility 128 (4 MB) | Optional AirPort | 🟡 Good |
| iBook G3/366 (Blueberry) | G3 366 MHz | 576 MB | ATI Rage Mobility 128 (4 MB) | Optional AirPort | 🟡 Good |
| iBook G3/366 SE (Graphite) | G3 366 MHz | 576 MB | ATI Rage Mobility 128 (8 MB) | Optional AirPort | 🟡 Good |
| iBook G3/466 SE (Key Lime) | G3 466 MHz | 576 MB | ATI Rage Mobility 128 (8 MB) | Optional AirPort | 🟡 Good |

Notes: Low RAM limits desktop options to LXDE/Openbox. Clamshell hinges are fragile. ATI Rage driver (aty128fb) works well.

### iBook G3 (Dual USB / Snow) — 2001–2003

| Model | CPU | RAM (Max) | GPU | WiFi | Rating |
| --- | --- | --- | --- | --- | --- |
| iBook G3/500 (Dual USB) | G3 500 MHz | 640 MB | ATI Rage Mobility 128 (8 MB) | Optional AirPort | 🟢 Excellent |
| iBook G3/600 | G3 600 MHz | 640 MB | ATI Radeon 7500 (16 MB) | AirPort | 🟢 Excellent |
| iBook G3/700 | G3 700 MHz | 640 MB | ATI Radeon 7500 (16 MB) | AirPort | 🟢 Excellent |
| iBook G3/800 | G3 800 MHz | 640 MB | ATI Radeon 7500 (32 MB) | AirPort | 🟢 Excellent |
| iBook G3/900 | G3 900 MHz | 640 MB | ATI Radeon 7500 (32 MB) | AirPort | 🟢 Excellent |

Notes: Excellent Linux support. The Dual USB iBook is one of the best-supported PPC laptops. Use radeon driver for GPU.

### iBook G4 — 2003–2006

| Model | CPU | RAM (Max) | GPU | WiFi | Rating |
| --- | --- | --- | --- | --- | --- |
| iBook G4/800 (12”) | G4 800 MHz | 1.12 GB | ATI Radeon 9200 (32 MB) | AirPort Extreme | 🟢 Excellent |
| iBook G4/933 (12”) | G4 933 MHz | 1.12 GB | ATI Radeon 9200 (32 MB) | AirPort Extreme | 🟢 Excellent |
| iBook G4/1.0 (14”) | G4 1.0 GHz | 1.12 GB | ATI Radeon 9200 (32 MB) | AirPort Extreme | 🟢 Excellent |
| iBook G4/1.2 | G4 1.2 GHz | 1.5 GB | ATI Radeon 9200 (32 MB) | AirPort Extreme | 🟢 Excellent |
| iBook G4/1.33 | G4 1.33 GHz | 1.5 GB | ATI Radeon 9200 (64 MB) | AirPort Extreme | 🟢 Excellent |
| iBook G4/1.42 | G4 1.42 GHz | 1.5 GB | ATI Radeon 9200 (64 MB) | AirPort Extreme | 🟢 Excellent |

Notes: Best PPC laptop for Linux. Good RAM, solid GPU, AltiVec support. AirPort Extreme uses b43 driver with firmware.

## 💻 PowerBook Series

### PowerBook G3 (Lombard & Pismo) — 1999–2001

| Model | CPU | RAM (Max) | GPU | WiFi | Rating |
| --- | --- | --- | --- | --- | --- |
| PowerBook G3 (Lombard) | G3 333/400 MHz | 512 MB | ATI Rage LT Pro (4 MB) | Optional AirPort | 🟡 Good |
| PowerBook G3 (Pismo) | G3 400/500 MHz | 1 GB | ATI Rage Mobility 128 (8 MB) | Optional AirPort | 🟢 Excellent |

Notes: Pismo is a classic — up to 1 GB RAM via aftermarket upgrade. Lombard has limited VRAM.

### PowerBook G4 (Titanium) — 2001–2003

| Model | CPU | RAM (Max) | GPU | WiFi | Rating |
| --- | --- | --- | --- | --- | --- |
| PowerBook G4/400 (Ti) | G4 400 MHz | 1 GB | ATI Rage Mobility 128 (8 MB) | Optional AirPort | 🟡 Good |
| PowerBook G4/500 (Ti) | G4 500 MHz | 1 GB | ATI Rage Mobility 128 (8 MB) | Optional AirPort | 🟡 Good |
| PowerBook G4/550 (Ti) | G4 550 MHz | 1 GB | ATI Radeon Mobility 7500 (16 MB) | AirPort | 🟢 Excellent |
| PowerBook G4/667 (Ti) | G4 667 MHz | 1 GB | ATI Radeon Mobility 7500 (16 MB) | AirPort | 🟢 Excellent |
| PowerBook G4/800 (Ti) | G4 800 MHz | 1 GB | ATI Radeon Mobility 9000 (32 MB) | AirPort | 🟢 Excellent |
| PowerBook G4/867 (Ti) | G4 867 MHz | 1 GB | ATI Radeon Mobility 9000 (32 MB) | AirPort | 🟢 Excellent |
| PowerBook G4/1.0 (Ti) | G4 1.0 GHz | 1 GB | ATI Radeon Mobility 9000 (64 MB) | AirPort | 🟢 Excellent |

Notes: Titanium hinges are fragile. Display cable issues common. Linux runs great on these.

### PowerBook G4 (Aluminum) — 2003–2006

| Model | CPU | RAM (Max) | GPU | WiFi | Rating |
| --- | --- | --- | --- | --- | --- |
| PowerBook G4/1.0 (12” Al) | G4 1.0 GHz | 1.25 GB | NVIDIA GeForce FX Go5200 (32 MB) | AirPort Extreme | 🟢 Excellent |
| PowerBook G4/1.25 (15” Al) | G4 1.25 GHz | 2 GB | ATI Radeon 9600 (64 MB) | AirPort Extreme | 🟢 Excellent |
| PowerBook G4/1.33 (12”/15”) | G4 1.33 GHz | 1.25/2 GB | NVIDIA FX Go5200 / ATI 9700 | AirPort Extreme | 🟢 Excellent |
| PowerBook G4/1.5 (12”/15”/17”) | G4 1.5 GHz | 1.25/2 GB | NVIDIA FX Go5200 / ATI 9700 | AirPort Extreme | 🟢 Excellent |
| PowerBook G4/1.67 (15”/17”) | G4 1.67 GHz | 2 GB | ATI Radeon 9700 (128 MB) | AirPort Extreme | 🟢 Excellent |
| PowerBook G4/1.67 Hi-Res (15”/17”) | G4 1.67 GHz | 2 GB | ATI Radeon 9700 (128 MB) | AirPort Extreme | 🟢 Excellent |

Notes: Peak PPC laptop performance. 12” uses NVIDIA (nouveau driver); 15”/17” use ATI (radeon driver). All have excellent Linux support. 2 GB RAM models can run XFCE comfortably.

## 🖥️ iMac Series

### iMac G3 — 1999–2003

| Model | CPU | RAM (Max) | GPU | WiFi | Rating |
| --- | --- | --- | --- | --- | --- |
| iMac G3/266–333 (Rev A–D) | G3 266–333 MHz | 512 MB | ATI Rage IIc / Rage Pro (2–6 MB) | None (add-on) | 🟠 Fair |
| iMac G3/350–500 (Slot-loading) | G3 350–500 MHz | 1 GB | ATI Rage 128 VR/Pro (8 MB) | Optional AirPort | 🟡 Good |
| iMac G3/500–700 (2001) | G3 500–700 MHz | 1 GB | ATI Rage 128 Ultra (16 MB) | AirPort | 🟡 Good |

Notes: Early tray-loading models have limited VRAM and low RAM ceiling. Slot-loading models are much better. Use LXDE/Openbox for best experience.

### iMac G4 (Flat Panel) — 2002–2004

| Model | CPU | RAM (Max) | GPU | WiFi | Rating |
| --- | --- | --- | --- | --- | --- |
| iMac G4/700 (15”) | G4 700 MHz | 1 GB | NVIDIA GeForce2 MX (32 MB) | AirPort | 🟢 Excellent |
| iMac G4/800 (15”) | G4 800 MHz | 1 GB | NVIDIA GeForce2 MX (32 MB) | AirPort | 🟢 Excellent |
| iMac G4/1.0 (15”/17”) | G4 1.0 GHz | 1 GB | NVIDIA GeForce4 MX (32–64 MB) | AirPort Extreme | 🟢 Excellent |
| iMac G4/1.25 (17”/20”) | G4 1.25 GHz | 2 GB | NVIDIA GeForce FX 5200 Ultra (64 MB) | AirPort Extreme | 🟢 Excellent |

Notes: Beautiful design. NVIDIA GPUs work well with nouveau driver. The 20” model has excellent screen for its era. Up to 2 GB RAM.

### iMac G5 — 2004–2006

| Model | CPU | RAM (Max) | GPU | WiFi | Rating |
| --- | --- | --- | --- | --- | --- |
| iMac G5/1.6 (17”) | G5 1.6 GHz | 2 GB | NVIDIA GeForce FX 5200 Ultra (64 MB) | AirPort Extreme | 🟢 Excellent |
| iMac G5/1.8 (17”/20”) | G5 1.8 GHz | 2 GB | NVIDIA GeForce FX 5200 Ultra (64 MB) | AirPort Extreme | 🟢 Excellent |
| iMac G5/2.0 (17”/20”) | G5 2.0 GHz | 2.5 GB | ATI Radeon 9600 (128 MB) | AirPort Extreme | 🟢 Excellent |
| iMac G5/2.1 (20”) iSight | G5 2.1 GHz | 2.5 GB | ATI Radeon X600 (128 MB) | AirPort Extreme | 🟡 Good |

Notes: G5 iMacs run warm — thermal management (windfarm module) is critical. The iSight model has an integrated camera that does not work under Linux. Use 64-bit Debian (ppc64).

## 🗄️ Power Mac Series

### Power Mac G3 (Blue & White) — 1999

| Model | CPU | RAM (Max) | GPU | WiFi | Rating |
| --- | --- | --- | --- | --- | --- |
| Power Mac G3 B&W/300 | G3 300 MHz | 1 GB | ATI Rage 128 (16 MB) | None (PCI card) | 🟡 Good |
| Power Mac G3 B&W/350 | G3 350 MHz | 1 GB | ATI Rage 128 (16 MB) | None (PCI card) | 🟡 Good |
| Power Mac G3 B&W/400 | G3 400 MHz | 1 GB | ATI Rage 128 (16 MB) | None (PCI card) | 🟡 Good |
| Power Mac G3 B&W/450 | G3 450 MHz | 1 GB | ATI Rage 128 (16 MB) | None (PCI card) | 🟡 Good |

Notes: First “New World” tower. PCI slots allow GPU/WiFi upgrades. IDE bus is the performance bottleneck.

### Power Mac G4 — 1999–2004

| Model | CPU | RAM (Max) | GPU | WiFi | Rating |
| --- | --- | --- | --- | --- | --- |
| Power Mac G4 (PCI Graphics) | G4 350–500 MHz | 2 GB | ATI Rage 128 Pro (16 MB) | Optional AirPort | 🟡 Good |
| Power Mac G4 (AGP/Sawtooth) | G4 350–500 MHz | 2 GB | ATI Rage 128 Pro (16 MB) | Optional AirPort | 🟢 Excellent |
| Power Mac G4 (Gigabit Ethernet) | G4 400–500 MHz (Dual) | 2 GB | ATI Rage 128 Pro (16 MB) | Optional AirPort | 🟢 Excellent |
| Power Mac G4 (Digital Audio) | G4 466–733 MHz | 2 GB | ATI Rage 128 Pro / NVIDIA GeForce2 MX (32 MB) | Optional AirPort | 🟢 Excellent |
| Power Mac G4 (Quicksilver) | G4 733 MHz–1 GHz (Dual) | 2 GB | NVIDIA GeForce2 MX (64 MB) | Optional AirPort | 🟢 Excellent |
| Power Mac G4 (MDD) | G4 867 MHz–1.42 GHz (Dual) | 2 GB | NVIDIA GeForce4 MX / ATI Radeon 9000 | AirPort Extreme | 🟢 Excellent |
| Power Mac G4 (FW800) | G4 1.0–1.42 GHz (Dual) | 2 GB | ATI Radeon 9000 Pro (64 MB) | AirPort Extreme | 🟢 Excellent |

Notes: Dual-processor models work great with SMP-enabled kernels. AGP slot allows GPU upgrades (Radeon 9200 recommended). MDD models are infamously loud (fan mod recommended).

### Power Mac G5 — 2003–2006

| Model | CPU | RAM (Max) | GPU | WiFi | Rating |
| --- | --- | --- | --- | --- | --- |
| Power Mac G5/1.6 | G5 1.6 GHz | 4 GB | NVIDIA GeForce FX 5200 Ultra (64 MB) | Optional AirPort Extreme | 🟢 Excellent |
| Power Mac G5/1.8 | G5 1.8 GHz | 8 GB | NVIDIA GeForce FX 5200 Ultra (64 MB) | Optional AirPort Extreme | 🟢 Excellent |
| Power Mac G5/2.0 (Dual) | G5 2.0 GHz ×2 | 8 GB | NVIDIA GeForce FX 5200 Ultra (64 MB) | Optional AirPort Extreme | 🟢 Excellent |
| Power Mac G5/2.5 (Dual) | G5 2.5 GHz ×2 | 8 GB | ATI Radeon 9600 XT (128 MB) | AirPort Extreme | 🟢 Excellent |
| Power Mac G5/2.7 (Dual) | G5 2.7 GHz ×2 | 8 GB | ATI Radeon 9600 XT (128 MB) | AirPort Extreme | 🟡 Good |
| Power Mac G5 Quad | G5 2.5 GHz ×4 | 16 GB | NVIDIA GeForce 6600 LE (256 MB) | AirPort Extreme | 🟡 Good |

Notes: Most powerful PPC Macs. Liquid-cooled 2.5/2.7 GHz models are prone to coolant leaks — check before buying! Use Debian ppc64 for 64-bit support. Quad requires windfarm thermal management. 8–16 GB RAM makes these viable desktop machines.

## 📦 Mac mini

### Mac mini G4 — 2005–2006

| Model | CPU | RAM (Max) | GPU | WiFi | Rating |
| --- | --- | --- | --- | --- | --- |
| Mac mini G4/1.25 | G4 1.25 GHz | 1 GB | ATI Radeon 9200 (32 MB) | AirPort Extreme + Bluetooth | 🟢 Excellent |
| Mac mini G4/1.42 | G4 1.42 GHz | 1 GB | ATI Radeon 9200 (32 MB) | AirPort Extreme + Bluetooth | 🟢 Excellent |
| Mac mini G4/1.5 | G4 1.5 GHz | 1 GB | ATI Radeon 9200 (64 MB) | AirPort Extreme + Bluetooth | 🟢 Excellent |

Notes: Excellent low-power PPC Linux machine. Quiet, small form factor. 1 GB RAM is limiting — use XFCE or LXDE. Great as a home server.

## 🖥️ eMac

### eMac — 2002–2006

| Model | CPU | RAM (Max) | GPU | WiFi | Rating |
| --- | --- | --- | --- | --- | --- |
| eMac G4/700 | G4 700 MHz | 1 GB | NVIDIA GeForce2 MX (32 MB) | Optional AirPort | 🟢 Excellent |
| eMac G4/800 | G4 800 MHz | 1 GB | ATI Radeon 7500 (32 MB) | Optional AirPort | 🟢 Excellent |
| eMac G4/1.0 | G4 1.0 GHz | 1 GB | ATI Radeon 7500 (32 MB) | AirPort Extreme | 🟢 Excellent |
| eMac G4/1.25 | G4 1.25 GHz | 2 GB | ATI Radeon 9200 (32 MB) | AirPort Extreme | 🟢 Excellent |
| eMac G4/1.42 | G4 1.42 GHz | 2 GB | ATI Radeon 9600 (64 MB) | AirPort Extreme | 🟢 Excellent |

Notes: CRT display — heavy but reliable. Excellent Linux support across the board. Later models with 2 GB RAM run XFCE well.

## 🖧 Xserve

### Xserve G4 / G5 — 2002–2006

| Model | CPU | RAM (Max) | GPU | WiFi | Rating |
| --- | --- | --- | --- | --- | --- |
| Xserve G4/1.0 (Single/Dual) | G4 1.0 GHz | 2 GB | ATI Rage 128 (headless) | None | 🟡 Good |
| Xserve G4/1.33 (Dual) | G4 1.33 GHz | 2 GB | ATI Radeon 9200 | None | 🟡 Good |
| Xserve G5/2.0 (Single/Dual) | G5 2.0 GHz | 8 GB | NVIDIA GeForce FX 5200 | None | 🟡 Good |
| Xserve G5/2.3 (Dual) | G5 2.3 GHz | 8 GB | ATI Radeon 9200 | None | 🟡 Good |

Notes: Server hardware — designed for headless operation. No WiFi. Serial console recommended for installation. Fibre Channel cards may not work under Linux.

## 📊 Driver Availability Matrix

| Component | Driver | G3 | G4 | G5 | Notes |
| --- | --- | --- | --- | --- | --- |
| CPU | Kernel PPC support | ✅ | ✅ | ✅ | CONFIG_PPC_PMAC=y |
| AltiVec / VMX | Kernel | ❌ | ✅ | ✅ | G3 lacks AltiVec |
| SMP (Dual CPU) | Kernel | ❌ | ✅ | ✅ | CONFIG_SMP=y needed |
| ATI Rage 128 | aty128fb / r128 | ✅ | ✅ | — | 2D only; no 3D accel |
| ATI Radeon (7x00–9x00) | radeon / radeonfb | ✅ | ✅ | ✅ | 3D acceleration works |
| ATI Radeon X-series | radeon | — | — | ✅ | DRM/KMS supported |
| NVIDIA GeForce2/4/FX | nouveau / nvidiafb | — | ✅ | ✅ | nouveau for 3D; nvidiafb for 2D only |
| NVIDIA GeForce 6xxx | nouveau | — | — | ✅ | G5 Quad only |
| AirPort (Original) | b43legacy | ✅ | ✅ | — | BCM4301/4306; needs firmware |
| AirPort Extreme | b43 | — | ✅ | ✅ | BCM4306 rev3+/4318/4311; needs firmware |
| Ethernet (Wired) | sungem / gem | ✅ | ✅ | ✅ | Works out of the box |
| Sound (Screamer/Burgundy) | snd-powermac | ✅ | ✅ | — | Older Macs |
| Sound (TAS/Toonie) | snd-aoa | — | ✅ | ✅ | Newer G4/G5 Macs |
| USB | ohci-hcd / ehci-hcd | ✅ | ✅ | ✅ | USB 1.1 (G3) / USB 2.0 (G4+) |
| FireWire | ohci1394 / firewire-ohci | ✅ | ✅ | ✅ | Works; some devices flaky |
| IDE/ATA | pata_macio / ide-pmac | ✅ | ✅ | ✅ | Standard for PPC Macs |
| SATA | sata_svw | — | — | ✅ | G5 only (Broadcom K2) |
| Thermal (G4 laptops) | therm_adt746x | — | ✅ | — | iBook G4, PowerBook G4 |
| Thermal (G5 desktop) | therm_pm72 / windfarm_* | — | — | ✅ | Critical for G5 |
| PMU (Power Management) | via-pmu | ✅ | ✅ | — | Sleep/lid close on laptops |
| Bluetooth | btusb | — | ✅ | ✅ | BT 1.1/2.0 EDR |
| iSight Camera | ❌ | — | — | ❌ | Not supported on PPC |

## ⚠️ Known Issues by Model

### iBook G3 Clamshell

- RAM ceiling (576 MB) severely limits usable desktop environments

- ATI Rage Mobility has no 3D acceleration — use fbdev if aty128fb causes issues

- Sleep may not resume reliably — disable via power manager

### iMac G3 (Tray-loading)

- Very low VRAM (2–6 MB) — may struggle with compositing

- No USB boot on earliest models — must use CD

- CRT geometry may need manual adjustment via xrandr or xvidtune

### Power Mac G4 (MDD / Mirror Drive Doors)

- Extremely loud fans — community fan-mod guides available

- Dual-CPU models may have power supply issues after many years

### Power Mac G5 (Liquid-cooled)

- Coolant leak risk on 2.5/2.7 GHz Dual models — check before use!

- windfarm kernel module MUST be loaded or the system will throttle/shut down

- High power consumption (300–600W under load)

### iMac G5 (iSight)

- iSight camera does not work under Linux

- Known capacitor failure issues on logic boards (all iMac G5 models)

### PowerBook G4 (Titanium)

- Fragile display hinge — handle with care

- Display cable failure common on 667/800/867/1.0 GHz models

- AirPort antenna reception is poor due to titanium case

## 💡 Recommended Configurations

### Best for Beginners

| Use Case | Recommended Model | Why |
| --- | --- | --- |
| PPC Linux laptop | iBook G4/1.33 | Reliable, good RAM, great driver support |
| PPC Linux desktop | Mac mini G4/1.42 | Quiet, low power, compact |
| PPC Linux workstation | Power Mac G5/2.0 Dual | 8 GB RAM, dual CPU, air-cooled (no leak risk) |

### Best Performance

| Use Case | Recommended Model | Why |
| --- | --- | --- |
| Maximum PPC performance | Power Mac G5 Quad | 4 cores, up to 16 GB RAM |
| Best PPC laptop | PowerBook G4/1.67 (15” Al) | 2 GB RAM, 128 MB GPU, Hi-Res display |
| Best all-in-one | iMac G5/2.0 (20”) | Large display, good specs |

### Best for Lightweight / Server Use

| Use Case | Recommended Model | Why |
| --- | --- | --- |
| Home server | Mac mini G4 | Low power, small, quiet, headless-friendly |
| Retro browsing / light use | iBook G3/900 | Portable, good enough for web with Dillo/NetSurf |
| File / print server | Xserve G4/G5 | Rack-mountable, built for server duty |

## 🔧 Testing Your Hardware

Run the Darby Linux PPC hardware detection script to get a full compatibility report for your specific Mac:

sudo ./scripts/hardware/ppc_detect.sh --save

This generates a detailed report including: - Exact model identification - CPU generation and features - RAM and storage details - GPU and display capabilities - WiFi and sound chip identification - Overall compatibility rating - Recommended distribution and desktop environment

Darby Linux v1.2.0 — PowerPC Hardware Compatibility List