# 💻 Supported Mac Models — Lightweight Linux for Macs

All Intel-based Macs from 2008 to 2019 are supported. Apple Silicon (M1/M2/M3/M4) Macs are not supported.

## Support Tiers

| Tier | Meaning |
| --- | --- |
| ✅ Full | All hardware works out of the box with this kit |
| 🟡 Good | Most hardware works; minor issues with one or two components |
| 🟠 Partial | Core functionality works; some hardware requires manual configuration |

## MacBook Pro

| Model Identifier | Year | Display | Tier | Notes |
| --- | --- | --- | --- | --- |
| MacBookPro5,1 | 2008 | 15” | ✅ Full | — |
| MacBookPro5,2 | 2009 | 17” | ✅ Full | — |
| MacBookPro5,3 | 2009 | 15” | ✅ Full | — |
| MacBookPro5,4 | 2009 | 15” | ✅ Full | — |
| MacBookPro5,5 | 2009 | 13” | ✅ Full | — |
| MacBookPro6,1 | 2010 | 17” | ✅ Full | — |
| MacBookPro6,2 | 2010 | 15” | ✅ Full | — |
| MacBookPro7,1 | 2010 | 13” | ✅ Full | — |
| MacBookPro8,1 | 2011 | 13” | ✅ Full | — |
| MacBookPro8,2 | 2011 | 15” | 🟡 Good | AMD GPU may need radeon.dpm=0 kernel param |
| MacBookPro8,3 | 2011 | 17” | 🟡 Good | AMD GPU may need radeon.dpm=0 kernel param |
| MacBookPro9,1 | 2012 | 15” | ✅ Full | — |
| MacBookPro9,2 | 2012 | 13” | ✅ Full | — |
| MacBookPro10,1 | 2012 | 15” Retina | ✅ Full | HiDPI auto-configured |
| MacBookPro10,2 | 2013 | 13” Retina | ✅ Full | HiDPI auto-configured |
| MacBookPro11,1 | 2013 | 13” Retina | ✅ Full | — |
| MacBookPro11,2 | 2013 | 15” Retina | ✅ Full | — |
| MacBookPro11,3 | 2013 | 15” Retina | 🟡 Good | NVIDIA GPU — use nouveau or proprietary |
| MacBookPro11,4 | 2015 | 15” Retina | ✅ Full | — |
| MacBookPro11,5 | 2015 | 15” Retina | 🟡 Good | AMD GPU — generally works well |
| MacBookPro12,1 | 2015 | 13” Retina | ✅ Full | — |
| MacBookPro13,1 | 2016 | 13” | 🟡 Good | SPI keyboard/trackpad driver needed |
| MacBookPro13,2 | 2016 | 13” Touch Bar | 🟠 Partial | Touch Bar basic support; SPI drivers needed |
| MacBookPro13,3 | 2016 | 15” Touch Bar | 🟠 Partial | Touch Bar basic support; SPI drivers needed |
| MacBookPro14,1 | 2017 | 13” | 🟡 Good | SPI keyboard/trackpad driver needed |
| MacBookPro14,2 | 2017 | 13” Touch Bar | 🟠 Partial | Touch Bar basic support |
| MacBookPro14,3 | 2017 | 15” Touch Bar | 🟠 Partial | Touch Bar basic support |
| MacBookPro15,1 | 2018 | 15” Touch Bar | 🟠 Partial | T2 chip requires workarounds; Touch Bar basic |
| MacBookPro15,2 | 2018 | 13” Touch Bar | 🟠 Partial | T2 chip requires workarounds |
| MacBookPro15,3 | 2019 | 15” Touch Bar | 🟠 Partial | T2 chip; use t2linux.org patches |
| MacBookPro15,4 | 2019 | 13” | 🟠 Partial | T2 chip |
| MacBookPro16,1 | 2019 | 16” | 🟠 Partial | T2 chip; AMD GPU |
| MacBookPro16,2 | 2019 | 13” | 🟠 Partial | T2 chip |

## MacBook Air

| Model Identifier | Year | Display | Tier | Notes |
| --- | --- | --- | --- | --- |
| MacBookAir2,1 | 2008 | 13” | ✅ Full | — |
| MacBookAir3,1 | 2010 | 11” | ✅ Full | — |
| MacBookAir3,2 | 2010 | 13” | ✅ Full | — |
| MacBookAir4,1 | 2011 | 11” | ✅ Full | — |
| MacBookAir4,2 | 2011 | 13” | ✅ Full | — |
| MacBookAir5,1 | 2012 | 11” | ✅ Full | — |
| MacBookAir5,2 | 2012 | 13” | ✅ Full | — |
| MacBookAir6,1 | 2013 | 11” | ✅ Full | — |
| MacBookAir6,2 | 2013 | 13” | ✅ Full | — |
| MacBookAir7,1 | 2015 | 11” | ✅ Full | — |
| MacBookAir7,2 | 2015 | 13” | ✅ Full | — |
| MacBookAir8,1 | 2018 | 13” Retina | 🟡 Good | T2 chip (limited); Retina auto-configured |
| MacBookAir8,2 | 2019 | 13” Retina | 🟡 Good | T2 chip |

## MacBook

| Model Identifier | Year | Display | Tier | Notes |
| --- | --- | --- | --- | --- |
| MacBook5,1 | 2008 | 13” | ✅ Full | — |
| MacBook5,2 | 2009 | 13” | ✅ Full | — |
| MacBook6,1 | 2009 | 13” | ✅ Full | — |
| MacBook7,1 | 2010 | 13” | ✅ Full | — |
| MacBook8,1 | 2015 | 12” Retina | 🟡 Good | SPI drivers needed |
| MacBook9,1 | 2016 | 12” Retina | 🟡 Good | SPI drivers needed |
| MacBook10,1 | 2017 | 12” Retina | 🟡 Good | SPI drivers needed |

## iMac

| Model Identifier | Year | Display | Tier | Notes |
| --- | --- | --- | --- | --- |
| iMac9,1 | 2009 | 20”/24” | ✅ Full | — |
| iMac10,1 | 2009 | 21.5”/27” | ✅ Full | — |
| iMac11,1 | 2009 | 27” | ✅ Full | — |
| iMac11,2 | 2010 | 21.5” | ✅ Full | — |
| iMac11,3 | 2010 | 27” | ✅ Full | — |
| iMac12,1 | 2011 | 21.5” | ✅ Full | — |
| iMac12,2 | 2011 | 27” | 🟡 Good | AMD GPU |
| iMac13,1 | 2012 | 21.5” | ✅ Full | — |
| iMac13,2 | 2012 | 27” | ✅ Full | — |
| iMac14,1 | 2013 | 21.5” | ✅ Full | — |
| iMac14,2 | 2013 | 27” | ✅ Full | — |
| iMac14,3 | 2013 | 21.5” | ✅ Full | — |
| iMac14,4 | 2014 | 21.5” | ✅ Full | — |
| iMac15,1 | 2014 | 27” 5K | 🟡 Good | 5K display may default to lower res |
| iMac16,1 | 2015 | 21.5” | ✅ Full | — |
| iMac16,2 | 2015 | 21.5” 4K | ✅ Full | — |
| iMac17,1 | 2015 | 27” 5K | 🟡 Good | AMD GPU; 5K via MST |
| iMac18,1 | 2017 | 21.5” | ✅ Full | — |
| iMac18,2 | 2017 | 21.5” 4K | ✅ Full | — |
| iMac18,3 | 2017 | 27” 5K | 🟡 Good | AMD GPU |
| iMac19,1 | 2019 | 27” 5K | 🟡 Good | T2 chip (limited) |
| iMac19,2 | 2019 | 21.5” 4K | 🟡 Good | T2 chip (limited) |

## Mac Mini

| Model Identifier | Year | Tier | Notes |
| --- | --- | --- | --- |
| Macmini3,1 | 2009 | ✅ Full | — |
| Macmini4,1 | 2010 | ✅ Full | — |
| Macmini5,1 | 2011 | ✅ Full | — |
| Macmini5,2 | 2011 | ✅ Full | — |
| Macmini5,3 | 2011 | ✅ Full | — |
| Macmini6,1 | 2012 | ✅ Full | — |
| Macmini6,2 | 2012 | ✅ Full | — |
| Macmini7,1 | 2014 | ✅ Full | — |
| Macmini8,1 | 2018 | 🟡 Good | T2 chip (limited) |

## Mac Pro

| Model Identifier | Year | Tier | Notes |
| --- | --- | --- | --- |
| MacPro3,1 | 2008 | ✅ Full | Great Linux workstation |
| MacPro4,1 | 2009 | ✅ Full | Great Linux workstation |
| MacPro5,1 | 2010–2012 | ✅ Full | The “cheese grater” — excellent Linux support |
| MacPro6,1 | 2013 | 🟡 Good | “Trash can” — dual AMD GPUs |
| MacPro7,1 | 2019 | 🟡 Good | T2 chip; AMD GPU |

## Notes on T2 Chip Macs (2018–2019)

Macs with the Apple T2 Security Chip require additional steps:

- Disable Secure Boot: Boot into macOS Recovery → Startup Security Utility → Set to “No Security”

- Allow external boot: Set to “Allow booting from external media”

- Install T2 Linux patches: Visit t2linux.org for kernel patches and instructions

- Internal SSD: The T2 chip manages the internal SSD; Linux support varies. An external USB SSD may be more reliable.

## Not Supported

| Device | Reason |
| --- | --- |
| Apple Silicon Macs (M1/M2/M3/M4) | Different CPU architecture (ARM); use Asahi Linux instead |
| PowerPC Macs (pre-2006) | Different architecture; use Debian PPC or Lubuntu PPC |
| MacBook 2006–2007 (32-bit) | Limited 32-bit EFI boot; may work with workarounds |