# 🔧 Troubleshooting Guide — Darby Linux

## Table of Contents

- WiFi Issues

- Graphics / Display Issues

- Audio Issues

- Trackpad Issues

- Keyboard Issues

- Bluetooth Issues

- Boot Issues

- Performance Issues

- Suspend / Resume Issues

- T2 Chip Issues

- Gaming Issues

- Security Issues

- Wallpaper Issues

- Update System Issues

- Getting Help

## WiFi Issues

### WiFi adapter not detected

Symptoms: No WiFi option in network settings, iwconfig shows no wireless interfaces.

Solution:

# Check what Broadcom chip you have
lspci -nn | grep -i network

# Reinstall drivers
sudo ./scripts/hardware/mac_drivers.sh --component wifi

# Try different driver combinations:

# Option 1: brcmfmac (modern, open-source — best for most Macs)
sudo modprobe -r wl b43 2>/dev/null
sudo modprobe brcmfmac

# Option 2: wl (Broadcom proprietary — broadcom-sta)
sudo modprobe -r brcmfmac b43 2>/dev/null
sudo modprobe wl

# Option 3: b43 (open-source — for older chips)
sudo modprobe -r wl brcmfmac 2>/dev/null
sudo modprobe b43

# After finding the working driver, reboot
sudo reboot

### WiFi connects but no internet

# Check DNS
ping -c 3 8.8.8.8          # If this works, it's a DNS issue
ping -c 3 google.com        # If this fails, set DNS manually

# Set Google DNS
echo "nameserver 8.8.8.8" | sudo tee /etc/resolv.conf
sudo systemctl restart NetworkManager

### WiFi drops after suspend

The kit includes a suspend/resume fix. If it’s not working:

# Verify the fix is installed
ls -la /usr/lib/systemd/system-sleep/99-mac-suspend-fix.sh

# Reinstall it
sudo cp configs/network/99-mac-suspend-fix.sh /usr/lib/systemd/system-sleep/
sudo chmod +x /usr/lib/systemd/system-sleep/99-mac-suspend-fix.sh

## Graphics / Display Issues

### Screen tearing

# For Intel GPUs — verify TearFree is enabled
cat /etc/X11/xorg.conf.d/20-intel.conf
# Should contain: Option "TearFree" "true"

# If file is missing, create it
sudo mkdir -p /etc/X11/xorg.conf.d
sudo bash -c 'cat > /etc/X11/xorg.conf.d/20-intel.conf << EOF
Section "Device"
    Identifier "Intel Graphics"
    Driver "intel"
    Option "TearFree" "true"
    Option "AccelMethod" "sna"
EndSection
EOF'

### Black screen after boot

# Boot into recovery mode (hold Shift during GRUB)
# Or add "nomodeset" to kernel parameters:

# At GRUB menu, press 'e' to edit
# Find the line starting with "linux" and add "nomodeset" before "quiet"
# Press Ctrl+X to boot

# Once booted, make it permanent:
sudo nano /etc/default/grub
# Change: GRUB_CMDLINE_LINUX_DEFAULT="quiet splash"
# To:     GRUB_CMDLINE_LINUX_DEFAULT="quiet splash nomodeset"
sudo update-grub

### Wrong resolution / scaling on Retina display

# Check current DPI
xdpyinfo | grep resolution

# Manually set DPI in XFCE
xfconf-query -c xsettings -p /Xft/DPI -s 144  # Adjust value as needed

# Or edit the HiDPI script
sudo nano /etc/profile.d/hidpi-xfce.sh

## Audio Issues

### No sound

# Check if audio devices are detected
aplay -l

# Check PulseAudio
pulseaudio --check && echo "Running" || echo "Not running"

# Restart PulseAudio
pulseaudio -k && pulseaudio --start

# Check ALSA mixer levels
alsamixer
# Use arrow keys to adjust, press 'M' to unmute

# Try different audio model for Apple hardware
sudo nano /etc/modprobe.d/apple-audio.conf
# Try these models one at a time:
#   options snd-hda-intel model=mbp101
#   options snd-hda-intel model=mbp55
#   options snd-hda-intel model=imac27_122
#   options snd-hda-intel model=auto
# Reboot after each change

### Headphone jack not working

# Install hdajackretask (allows remapping audio ports)
sudo apt install alsa-tools-gui
hdajackretask

## Trackpad Issues

### Trackpad not responding

# Check if the trackpad is detected
xinput list | grep -i "touch\|track\|bcm"

# Check kernel modules
lsmod | grep -i "bcm5974\|applespi\|hid_apple"

# Load module manually
sudo modprobe bcm5974  # For USB trackpads
sudo modprobe applespi # For SPI trackpads (2015+ MacBooks)

### Trackpad too sensitive / not sensitive enough

# Adjust acceleration speed in the config
sudo nano /etc/X11/xorg.conf.d/70-apple-trackpad.conf
# Change AccelSpeed value (range: -1.0 to 1.0)
# Higher = faster/more sensitive

### No tap-to-click

# Verify the config has tapping enabled
grep -i "tapping" /etc/X11/xorg.conf.d/70-apple-trackpad.conf
# Should show: Option "Tapping" "on"

# Apply changes without reboot
sudo systemctl restart lightdm

## Keyboard Issues

### Function keys not working as expected

# Check current fnmode
cat /sys/module/hid_apple/parameters/fnmode
# 0 = disabled, 1 = media first, 2 = function first

# Change temporarily
echo 2 | sudo tee /sys/module/hid_apple/parameters/fnmode

# Change permanently
sudo nano /etc/modprobe.d/apple-keyboard.conf
# Set: options hid_apple fnmode=2
sudo update-initramfs -u

### Command (⌘) and Alt/Option (⌥) swapped

# The kit swaps them by default for PC-like layout
# To restore Mac layout:
sudo nano /etc/modprobe.d/apple-keyboard.conf
# Change: options hid_apple swap_opt_cmd=0
sudo update-initramfs -u
sudo reboot

## Bluetooth Issues

### Bluetooth not finding devices

# Check Bluetooth service
sudo systemctl status bluetooth

# Restart Bluetooth
sudo systemctl restart bluetooth

# Check if adapter is blocked
rfkill list bluetooth
sudo rfkill unblock bluetooth

# Scan for devices
bluetoothctl
> power on
> agent on
> scan on

### Bluetooth audio stuttering

# Edit PulseAudio Bluetooth config
sudo nano /etc/pulse/default.pa
# Add or modify:
# load-module module-bluetooth-discover

# Restart PulseAudio
pulseaudio -k && pulseaudio --start

## Boot Issues

### GRUB menu not showing

# Hold Shift during boot to show GRUB
# To make it always visible:
sudo nano /etc/default/grub
# Change: GRUB_TIMEOUT_STYLE=hidden → GRUB_TIMEOUT_STYLE=menu
# Change: GRUB_TIMEOUT=0 → GRUB_TIMEOUT=5
sudo update-grub

### Mac won’t boot from USB

- Shut down completely (don’t restart)

- Press power button, then immediately hold Option (⌥)

- Wait for the boot disk selection screen

- Select the USB drive

If the USB doesn’t appear: - Try a different USB port - Recreate the USB with a different tool - On newer Macs, you may need to disable Secure Boot in macOS Recovery

### Boot stuck at black screen with cursor

This usually means the display driver is failing. See Graphics / Display Issues.

## Performance Issues

### System feels slow

# Check resource usage
htop

# Verify performance tweaks are applied
cat /etc/sysctl.d/99-mac-performance.conf

# Check if TLP is running
sudo tlp-stat -s

# Disable animations in XFCE
xfconf-query -c xfwm4 -p /general/use_compositing -s false

### High CPU temperature / fans always on

# Check temperature
sensors  # (install: sudo apt install lm-sensors && sudo sensors-detect)

# Check if thermald is running
sudo systemctl status thermald

# Install mbpfan for better fan control
sudo apt install mbpfan
sudo systemctl enable mbpfan
sudo systemctl start mbpfan

## Suspend / Resume Issues

### Mac won’t wake from suspend

# Try closing and opening the lid
# If that doesn't work, hold the power button for 5 seconds

# Disable problematic suspend method
sudo nano /etc/default/grub
# Add to GRUB_CMDLINE_LINUX_DEFAULT: acpi_osi=Darwin
sudo update-grub

### WiFi/Bluetooth dead after resume

See WiFi drops after suspend above.

## T2 Chip Issues

### Keyboard/trackpad not working on T2 Mac

# Check if apple-bce module is loaded
lsmod | grep apple_bce

# Load it manually
sudo modprobe apple-bce

# If module not found, install T2 support
sudo ./scripts/hardware/t2_support.sh --keyboard

# Or install the full t2linux kernel (recommended)
sudo ./scripts/hardware/t2_support.sh --kernel
sudo reboot

### No audio on T2 Mac

# Check T2 audio status
sudo ./scripts/hardware/t2_support.sh --check

# Reinstall T2 audio support
sudo ./scripts/hardware/t2_support.sh --audio

# Check if audio devices are detected
aplay -l

# The apple-bce module must be loaded for audio
lsmod | grep apple_bce

### Touch Bar shows nothing

# Check if driver is loaded
lsmod | grep apple_ib_tb

# Load manually
sudo modprobe apple-ibridge
sudo modprobe apple-ib-tb

# If not available, install Touch Bar support
sudo ./scripts/hardware/t2_support.sh --touchbar

# Change mode
sudo touchbar-mode 1    # Force function keys

### T2 Mac won’t boot Linux

- Ensure Secure Boot is disabled: Boot into macOS Recovery → Startup Security Utility → “No Security”

- Enable external boot media in the same utility

- Apply boot fixes: sudo ./scripts/hardware/t2_support.sh --boot

- If GRUB fails, try systemd-boot: sudo bootctl install

## Gaming Issues

### Steam won’t launch

# Check if 32-bit libraries are installed
dpkg --print-foreign-architectures  # Should show "i386"

# Reinstall Steam dependencies
sudo dpkg --add-architecture i386
sudo apt update
sudo apt install -y libc6:i386 libstdc++6:i386 libgl1:i386

# Reinstall Steam
sudo ./scripts/gaming/setup_gaming.sh --steam

### Games crash with Proton

# Check Vulkan support
vulkaninfo --summary

# Ensure Vulkan drivers are installed
sudo apt install mesa-vulkan-drivers mesa-vulkan-drivers:i386

# Try a different Proton version in Steam:
# Steam → Game → Properties → Compatibility → Select Proton version

# Check gaming kernel parameter
cat /proc/sys/vm/max_map_count
# Should be 2147483642 for gaming. If not:
echo 2147483642 | sudo tee /proc/sys/vm/max_map_count

### Poor gaming performance

# Switch to performance governor
sudo ./scripts/gaming/setup_gaming.sh --performance-on

# Use gamemode launcher
gamemoderun steam

# Check GPU driver status
sudo ./scripts/gaming/setup_gaming.sh --status

# For NVIDIA: Install proprietary drivers
sudo apt install nvidia-driver nvidia-settings

## Security Issues

### Locked out by firewall

# If you have physical access:
sudo ufw disable
sudo ufw status

# Re-enable with safe rules
sudo ufw reset
sudo ufw default deny incoming
sudo ufw default allow outgoing
sudo ufw allow ssh
sudo ufw enable

### Fail2ban blocked my IP

# Check banned IPs
sudo fail2ban-client status sshd

# Unban a specific IP
sudo fail2ban-client set sshd unbanip YOUR_IP_ADDRESS

# Check jail status
sudo fail2ban-client status

### Can’t SSH after hardening

The SSH hardening script does not disable password authentication by default. If you manually enabled key-only auth without setting up keys:

# From physical console:
sudo nano /etc/ssh/sshd_config.d/90-darby-hardened.conf
# Make sure PasswordAuthentication is NOT set to "no"
# Or comment it out
sudo systemctl restart sshd

## Wallpaper Issues

### Wallpaper not generated

# Check for required tools
which convert     # ImageMagick
which python3     # Python 3

# Install ImageMagick if missing
sudo apt install imagemagick

# Regenerate
sudo ./scripts/ui/wallpaper_manager.sh --generate

### Wallpaper doesn’t fit screen

# Check your resolution
xrandr | grep " connected"

# Regenerate at your resolution
sudo ./scripts/ui/wallpaper_manager.sh --generate

# Or scale an existing wallpaper
sudo ./scripts/ui/wallpaper_manager.sh --scale /usr/share/darby-linux/wallpapers/darby-night-sky.png 2560x1440

### Wallpaper not applied to all monitors

# Check monitor info
./scripts/ui/wallpaper_manager.sh --info

# Set per-monitor wallpaper
./scripts/ui/wallpaper_manager.sh --set-monitor 1 /path/to/wallpaper.png
./scripts/ui/wallpaper_manager.sh --set-monitor 2 /path/to/other.png

## Update System Issues

### OTA check fails

# Check network connectivity
curl -I https://github.com

# Check update configuration
cat /etc/lightweight-linux-mac/update.conf

# Run check manually with verbose output
sudo bash -x /opt/lightweight-linux-mac/scripts/ota/update_manager.sh check

### Rollback fails

# List available backups
ls -la /var/cache/lightweight-linux-mac/rollback/

# Manual rollback
sudo /opt/lightweight-linux-mac/scripts/ota/update_manager.sh rollback

## Getting Help

If your issue isn’t listed here:

- Check logs: cat /var/log/lightweight-linux-mac-install.log

- Search the issue tracker: GitHub Issues

- Open a new issue with:

  - Your Mac model (cat /etc/lightweight-linux-mac/hardware.conf)

  - Linux distribution and version (lsb_release -a)

  - Kernel version (uname -r)

  - Description of the problem and steps to reproduce

  - Relevant log output